- Product Specification

Part NO./产品型号：   GX_USB2MIPI30P_21X_V2.0

Product Code/产品编码：       A00727

| 制定 | 审核 | 核准 |
| --- | --- | --- |
| Sandy.Hu | Peter.Pan | Thanch.Li |

公司地址：深圳市光明区玉塘街道田阔路15号3楼

电话： 15989334751

邮箱：gaoxin202202@163.com

CONTENTS / 目录

Product Specification	1

1.General Description/概述	3

2.Features/特性	3

3.Interface Function/接口功能	4

4.Interface Descriptions/接口定义	5

4.1   CN4：MIPI Output (0.5mm 30pin FPC)	5

4.2   CN1：5V & USB Input (标准16Pin Type-C)	7

4.3   CN5：Key Board Interface (1.25mm 8pin)	8

5.Mechanical Specifications/结构尺寸	9

6.Operating & Storage Range/工作存储条件	10

7. Notes/注意事项	10

8.Revision History/修改记录	10

# 1.General Description/概述

GX_USB2MIPI30P_21X_V2.0 is a high performance USB-to-MIPI video converter. It integrates an USB2.0 receiver and a MIPI-DSI transmitter. Horizontal and vertical flips, 90/180/270 rotates are supported, It also integrates a IIC interface for Capacitive Touch Panel. MIPI-DSI supports 1080P@60Hz Max.

GX_USB2MIPI30P801_V2.0是一款高性能USB转MIPI视频转接板，集成一个USB2.0输入，MIPI-DSI输出。支持水平和垂直翻转，支持90/180/270旋转，支持IIC触摸，MIPI最高分辨率是1920x1080@60Hz。

# 2.Features/特性

| Function/功能 | Description/描述 |
| --- | --- |
| Signal Input/信号输入 | USB2.0 (Type-C) |
| Signal Output/信号输出 | MIPI DSI (4 lane) |
| Power Supply/电源 | 5V DC，单板工作时0.8W，受限于主机USB输出功率，所选屏的功耗不能太大 |
| Backlight/背光 | 180mA Max/最大180mA(可根据屏背光更改) |

# 3.Interface Function/接口功能

| Connector No./位号 | Descriptions/描述 |
| --- | --- |
| CN1 | 5V & USB Input / 5V 和 USB 输入 |
| CN5 | Key Board Interface / 按键接口 |
| CN4 | MIPI output / MIPI 输出 |

# 4.Interface Descriptions/接口定义

- 4.1   CN4：MIPI Output (0.5mm 30pin FPC)

| Pin NO./编号 | Symbol/定义 | Description/功能描述 |
| --- | --- | --- |
| 1 | NC | NC |
| 2 | TP_VDD | TP VDD 3.3V |
| 3 | TP_RST | TP RST |
| 4 | TP_SCL | TP SCL |
| 5 | TP_SDA | TP SDA |
| 6 | TP_INT | TP INT |
| 7 | GND | Ground |
| 8 | MIPI_D0P | MIPI Data Lane 0 Positive Output Pin |
| 9 | MIPI_D0N | MIPI Data Lane 0 Negative Output Pin |
| 10 | GND | Ground |
| 11 | MIPI_D1P | MIPI Data Lane 1 Positive Output Pin |
| 12 | MIPI_D1N | MIPI Data Lane 1 Negative Output Pin |
| 13 | GND | Ground |
| 14 | MIPI_CLKP | MIPI Clock Lane Positive Output Pin |
| 15 | MIPI_CLKN | MIPI Clock Lane Negative Output Pin |
| 16 | GND | Ground |
| 17 | MIPI_D2P | MIPI Data Lane 2 Positive Output Pin |
| 18 | MIPI_D2N | MIPI Data Lane 2 Negative Output Pin |
| 19 | GND | Ground |
| 20 | MIPI_D3P | MIPI Data Lane 3 Positive Output Pin |
| 21 | MIPI_D3N | MIPI Data Lane 3 Negative Output Pin |
| 22 | GND | Ground |
| 23 | NC | NC |
| 24 | NC | NC |
| 25 | RESET | Panel Reset |
| 26 | VCC | Panel Power Supply |
| 27 | VCC | Panel Power Supply |
| 28 | LEDK | Backlight Cathode |
| 29 | LEDK | Backlight Cathode |
| 30 | LEDA | Backlight Anode |

- 4.2   CN1：5V & USB Input (标准16Pin Type-C)

- 4.3   CN5：Key Board Interface (1.25mm 8pin)

| Pin NO./编号 | Symbol/定义 | Description/功能描述 |
| --- | --- | --- |
| 1 | K4 | 菜单/确认 |
| 2 | K3 | 退出 |
| 3 | K2 | 左移/下移/减 |
| 4 | K1 | 右移/上移/加 |
| 5 | GND | 地 |
| 6 | LED2 | 绿灯指示灯 |
| 7 | LED1 | 红灯指示灯 |
| 8 | K0 | 开关机 |

# 5.Mechanical Specifications/结构尺寸

Controlling Dimension: Millimeter(mm)

Tolerance：±0.3mm

# 6.Operating & Storage Range/工作存储条件

- Relative humidity/湿度（非凝露）: ≤ 80%

- 存储条件：温度0℃~+40℃；相对湿度5-85%

- 工作条件：温度0℃~+40℃；相对湿度20-80%

# 7. Notes/注意事项

- 保持电路板远离强静电。

- 注意检测连接线的方向及屏电压是否正确后再通电，否则会损坏LCD模块。

- 在电路板工作时，避免金属掉落在电路板中。

- 请勿在电路板工作时推拉连接器。

- 请勿拆解模块。

- 如果主板表面很脏，请用软布（非湿布）清洁。

# 8.Revision History/修改记录

| Revision No. | Date | Description |
| --- | --- | --- |
| V1.0 | 2024-10-08 | 初版 |
| V2.0 | 2025-01-03 | 修复IIC问题 |